SMARTGROWTH INFRASTRUCTURE CAPACITY

REPORT

ADDRESSING GENERATION 1 – 4 DEVELOPMENT.

October 2012

## EXECUTIVE SUMMARY:

1.1  Purpose

The purpose of this report is to address ability of the existing and planned infrastructure to service growth identified within the Urban Limits Line of the Regional Policy Statement, out to post 2021. It covers the SmartGrowth area which includes Western Bay of Plenty District and Tauranga City.

1.2  Background.

Growth within this area has been broken down into four areas known as Generation 1 – 4. These are described as follows:

| Generation | Period. |
| --- | --- |
| 1 | Pre 1990’s |
| 2 | 1990 - 2000 |
| 3 | 2001 - 2021 |
| 4 | 2021 - 2051 |

This report specifically addresses the Generation 4 development but where necessary comments on Generation 2 & 3.

1.3 Current Situation

Currently, all of the Generation 1, 2 & 3 areas have been rezoned to allow for residential, commercial and industrial development and these are all operative under the Tauranga and Western Bay of Plenty District Plans. Servicing of this land is fully planned or available.

All of the Generation 4 areas identified for future development are included within the current Urban Limits Line and shown as “post 2021” development. Of these, the Tauriko area has already been rezoned (Industrial) as this was done as part of the private plan change carried out by IMF Ltd for the whole of the Tauriko area. Omokoroa Stage 2 area has been zoned for Future Urban.

All other Generation 4 areas have had some work done towards their future servicing but none, except for Omokoroa Stage 2 and Tauriko Stage 2, have reached the stage of having structure plans prepared.

1.4 Generation 4 Areas.

The following table identifies the Generation 4 areas and the infrastructural requirements for future development.

| AREA | REQUIREMENTS. |
| --- | --- |
| Tauriko Stage 2 | Structure plan completed and development under way. Full water and wastewater services not available until 2016. Roading connection to SH No.29 at Belk Road needs further work. |
| Keenan Road | No structure plan yet. A new water reservoir will be required and approximately 7Km. of wastewater pipe would be required. Keenan Road would need to be upgraded to a residential collector standard and the Keenan Road/SH No. 36 intersection will also need to be upgraded. |
| Pukemapu Road. | No structure plan yet. A reservoir, fed from the Oropi Treatment plant, would be required at the top end of the area and a wastewater main will be required to be laid to Maleme Street pump station. Roading issues relate to the upgrading of the Pukemapu Road bridge and road between the bridge and Oropi Road. Work will be required to assess the effect of this development on the State Highway intersection with Oropi Road. |
| Neewood Road | No structure plan yet. A further reservoir will need to be constructed above the Ohauiti Road/Rowe Road intersection and this would be fed from the Pukemapu reservoir supply. Wastewater would be as an extension of the Pukemapu system. The effect of the extra traffic generated from this area will need to be assessed on Ohauiti Road, Poike Road and the State Highway. |
| Upper Ohauiti. | No structure plan yet. Water supply to most of this area will be available from the existing Ohauiti reservoir. Wastewater supply would be as an extension of the Pukemapu trunk main. Roading access would be by way of cul de sac’s off Ohauiti Road. The effect of the extra traffic generated from this area, as well as Neewood Road, will need to be assessed on Ohauiti Road, Poike Road and the State Highway. Traffic issues have the potential to become a fatal flaw. |
| Kaitemako Road. | Little work has been done on this however closer study of the land topography shows that much of the area is too steep for future residential development. It is therefore recommended that it be removed from the SmartGrowth Settlement Pattern. |
| Te Tumu | No structure plan yet but considerable work has been done towards the future development of this land. Work completed to date shows that development of this land would be viable. Water supply is planned to be provided from the future Waiari scheme. There are two potential wastewater systems that could be considered along with future Generation 5 options. Roading access wil be by way of the future Tauranga Eastern Motorway interchange at the end of Bell Road along with Te Okuroa Drive, The Boulevard and Papamoa Beach Road. |
| Waihi Beach Wilson Park Three Mile Creek Athenree. | No structure plan yet. Water supply will be by way of the existing Waihi Beach system as will the Wastewater disposal. No significant roading issues have been identified. |
| Katikati Busby Road Johnson Farm Tetley Road (Indust.) | No structure plans yet. Water supply from the Wharawhara Valley is sufficient for all identified development. Wastewater disposal has the potential to limit the amount of further growth in Katikati depending upon the outcome of the flushing of the existing outfall pipe from the Park Road treatment plant across the harbour and Matakana Island to the ocean discharge. Some roading connections to the State Highway may require further negotiation with NZTA. |
| Omokoroa. Stage 2 | No structure plans yet but area zoned “Future Urban”. Water supply is planned and wastewater is to connect to the trunk main from Omokoroa to Chapel Street Treatment Plant. Upgrading of the Omokoroa Road/SH No. 2 intersection will be required. |
| Te Puke. Cannel Farm No.3 Road Industrial | Some structure planning has been completed. Water supply will come from the existing bores in No.1 Road. Wastewater will be discharge to the existing Te Puke Treatment Plan but this will need upgrading. Further growth is likely to result in the need to upgrade No.1 Road, No.3 Road and Te Puke Quarry Road intersections with the State Highway. |

- CONCLUSIONS.

Conclusions to be drawn from this report are:

- Development of the Tauriko Stage 2 area will be restricted until 2016 when the Southern Pipeline and the second connection to the water supply are completed.

- Development of Keenan Road is dependant upon cost effective wastewater and roading solutions being achieved.

- A decision needs to be made as to whether Pukemapu and Neewood roads need to be linked.

- Pukemapu Road development will depend upon the viability of upgrading the Waimapu Bridge and connecting road from the bridge to Oropi Road and whatever upgrading may be required to the Oropi Road/SH No.29 intersection. This would be assisted if the link between Pukemapu Road and Neewood Road were made.

- Neewood Road could be viable but would be assisted by the development of Pukemapu Road first, thus facilitating more cost effective wastewater services.

- Upper Ohauiti does not have the same yield potential as Neewood Road and this area combined with Neewood Road could create traffic issues on Ohauiti Road and consequently the State Highway. If these problems prove significant, this area could be forfeited in favour of Neewood Road.

- Kaitemako Road land is steeper than first envisaged and the potential yield is low. It is quite isolated from other residential development and with the low yield is likely to be expensive to develop. This land is best left as “Rural” land and should be removed from the SmartGrowth settlement Pattern.

- Work done on the Te Tumu area shows that it is viable to develop.

- Waihi Beach. No impediments are envisaged for the development of this land.

- Katikati. The major impediment to the further development of Katikati is the capacity of the wastewater ocean outfall pipe. Given the current situation, the existing population exceeds the capacity of this pipe during storm events but work is being done to flush the pipe which has the potential to vastly improve its capacity.

- Water and wastewater services are available for the development of Omokoroa Stage 2 but the roading connection to State Highway No. 2 is still to be resolved.

- An upgrade of the Te Puke wastewater treatment plant is planned to increase its capacity to the maximum consented discharge for the equivalent of 12,000 people. SmartGrowth’s current projected population for the town is 13,000 people. NZTA is currently conducting a post TEL form and function review for the roading corridor through Te Puke.

- PURPOSE

The purpose of this report is to address ability of the existing and planned infrastructure to service growth identified within the Urban Limits Line of the Regional Policy Statement, out to post 2021. It covers the SmartGrowth area which includes Western Bay of Plenty District and Tauranga City.

- BACKGROUND:

In planning for the growth of Tauranga City and Western Bay of Plenty District since 1991, Urban Growth Areas have been created which identify the projected population for each area and the servicing required to meet the needs of that population. Servicing included water, wastewater, stormwater, roading and reserves and the approximate location of these services was shown on area structure plans. These first urban growth areas are referred to in this report as the Generation 1 areas and since then Generation 2 areas have been developed, with Generations 3 & 4 under consideration. These are illustrated on the following table.

| Generation | Areas Covered | Current Situation |
| --- | --- | --- |
| 1 (Pre 1990’s) | Waihi Beach, Katikati, Omokoroa, Tauranga City and Te Puke pre 1990’s | Largely developed now with some infill under way and intensification starting to take place. |
| 2 (1990 – 2000) | Tauranga. Bethlehem, Pyes Pa, Ohauiti, Welcome Bay and Papamoa. Katikati Shrewsbury Park. Waihi Beach Browns Farm. Kotunui Road. | Development of these areas is now nearing completion. |
| 3 (2001 – 2021) | Tauranga. Pyes Pa West, Tauriko Stage 1, West Bethlehem, Hastings Road, Kennedy Road and Wairakei Waihi Beach. Citrus Avenue, Athenree and Ocean Breeze. Omokoroa. Stage 1 (East of the Railway)and Pt. Stage 2 (Goldstone Blocks) Te Puke. McLoughlin Drive and Dudley Vercoe Drive. | These areas have been recently rezoned to allow for Residential, Commercial and Industrial development which is now starting to take place. |
| 4 (Post 2021 - 2051) | Tauranga. Tauriko Stage 2, Keenan Road, Pukemapu Road, Neewood Road, Upper Ohauiti, Kaitemako Road and Te Tumu. Waihi Beach. Wilson Farm, 3 Mile Creek and Athenree. Katikati. Busby Road, Johnson Farm (Park Road) Tetley Road Industrial. Omokoroa. Stage 2 (Railway – SH No. 2) Te Puke Cannel Farm South. | This is land that has been included within the Urban Limits Line for development post 2021. Except for Tauriko Stage 2 and Omokoroa Stage 2, no detailed structure planning work has been done for these areas yet but some “Big Picture” thinking has taken place. |
| 5 (Post 2051) | These areas are yet to be identified but are likely to be selected from the 16 areas currently identified in the SmartGrowth Land Capacity and Suitability report dated November 2011, as being worthy of further study. | Urban Growth Areas yet to be Identified. |

This report addresses the Generation 4 growth.

- CURRENT SITUATION:

Currently, all of the Generation 1, 2 & 3 areas have been rezoned to allow for residential, commercial and industrial development and these are all operative under the Tauranga and Western Bay of Plenty District Plans. Servicing of this land is fully planned or available.

All of the Generation 4 areas identified for future development are included within the current Urban Limits Line and shown as “post 2021” development. Of these, the Tauriko area has already been rezoned (Industrial) as this was done as part of the private plan change carried out by IMF Ltd for the whole of the Tauriko area. All of the services are planned for the area however a full wastewater system will not be available to Tauriko until after 2016 when the Southern Pipeline is intended to be completed through to the Te Maunga Wastewater Treatment Plant. Further growth in Tauriko is currently being restricted awaiting completion of this pipeline. There is also a limitation on the water supply as a new link is to be constructed from Joyce Road to Tauriko including a new reservoir to be located within the Lakes development. Without this link being constructed there is no security of supply as there is only one water pipe supplying the whole area at present. The new pipe is planned to be completed by 2016.

All other Generation 4 areas have had some work done towards their future servicing but none, except for Omokoroa Stage 2 and Tauriko Stage 2, have reached the stage of having structure plans prepared.

- GENERATION 4 AREAS.

# 6.1  Tauranga City

- Tauriko Stage 2

The structure plan for this area has been completed and infrastructure is being extended through Stage 1 towards Stage 2 as the demand requires.

  - Water: A second water supply from Kennedy Road is to be installed in 2015/16 along with a high level supply to service the Gargan Road plateau at the same time.

  - Wastewater: Wastewater servicing will be completed by 2016 when the Southern Pipeline is extended through to Te Maunga Wastewater Treatment Plant. Until that time further development will be restricted.

  - Stormwater: All stormwater treatment and detention ponds have been identified on the Tauriko Structure Plan. Some have been constructed.

  - Roading: As development extends towards Belk Road consideration will need to be given to what roading access will be required. Currently the structure plan allows for the upgrading of Belk Road back towards the existing SH No. 29 with extensive upgrading of that intersection, the nature of which is yet to be decided.

- Keenan Road.

No detailed work has been completed on this area yet but the major issues that have been identified for the future development of this area are;

- Water: There will be a need for a reservoir to be constructed on the higher land to the south which would be serviced from the Oropi Road Treatment Plant.

- Wastewater: The decision to include Keenan Road as a Generation 4 area was made after “The Lakes” subdivision had commenced and therefore the wastewater pipe sizes that service The Lakes subdivision were not sized for the extra flow that would result from development of Keenan Road. One solution available to this is to install a pump station and 5.5Km rising main to a pipe in Oropi Road that would service the Pukemapu Road area and then share the cost of a further 1.5Km of pipe from Pukemapu Road to the Maleme Street pump station. A second solution could be available should the industrial development on the south side of Belk Road proceed. This development would require a pump station to be built in the Belk Road South area and a rising main to be constructed from this pump station to the Maleme Street pump station. It would be possible to connect the Kennedy Road area into this pipeline, again giving a total distance of approximately 7Km.

- Stormwater : No work has been done on the stormwater requirements but, given the topography, no problems are envisaged. Detention and treatment ponds will be able to be located in the gully floors.

- Roading: Matters to be considered are the upgrading of Keenan Road, including its intersection with Pyes Pa Road (SH No.36), The need for roading connectivity between Keenan Road UGA and The Lakes subdivision.

Other considerations for the development of this area are the potential geotechnical constraints along the edges of the escarpments that run through the land.

- Pukemapu/Neewood Area

Note that the next four areas, Pukemapu Road, Neewood Road, Ohauiti South and Kaitemako Road, are all inter-reliant to some extent and it is therefore important to decide which of these areas are to proceed and those that should not. The main issues revolve around roading and wastewater provisions.

The roading issues are twofold. The first is the question of whether we should be planning for Pukemapu Road and Neewood Road to be linked together. This link provides the interconnectivity between Oropi and Ohauiti and would ensure a greater catchment to pay for the Waimapu Stream bridge and link to Oropi Road upgrading however, the second issue is the affect that this link will have on the Oropi Road, Poeke Road  and Welcome Bay Road intersections to State Highway No.29. There is also considerable doubt that Ohauiti Road could handle the potential traffic flows from Neewood Road and Upper Ohauiti without the need to be widened to four lanes. This is likely to be a fatal flaw due to the high cost of achieving this. Traffic modelling will be required to ensure that these issues are properly addressed.

From a wastewater perspective, all of these areas would need to be serviced from the trunk main that would service Pukemapu Road area first and then be extended to Neewood Road and Ohauiti South areas and then again from Upper Ohauiti to Kaitemako Road. Early decisions as to which of these areas are to develop will be required so that the correct pipe size for the trunk main can be determined. Also, if Upper Ohauiti area were not to proceed, then the servicing of Kaitemako Road becomes very expensive. These four areas are now addressed individually.

If the road link between Pukemapu Road and Neewood Road were not to proceed then the household yields would reduce as follows:

|  | AREA (Ha) | USEABLE % | USEABLE AREA (Ha) | COMMUNITY USE (7%) | DENSITY | No. OF HOUSES |
| --- | --- | --- | --- | --- | --- | --- |
| Neewood Rd. | 91 | 90 | 81.9 | 5.7 | 15 | 1143 |
| Pukemapu Rd. | 138 | 90 | 124.2 | 8.7 | 15 | 1732 |
| Ohauiti South | 80 | 90 | 72 | 5.0 | 15 | 1005 |
| Undeveloped Area | 59 | 90 | 53.1 | 3.7 | 15 | 741 |
| TOTALS | 368 | 90 | 331.2 | 23.1 | 15 | 4621 |

If the Pukemapu / Neewood link road were not established then the overall yield would drop from 4621 to 3880 households.

- Pukemapu Road.

- Water: The water supply to Pukemapu Road would come from the Oropi Water Treatment Plant and would need to be pumped up to a reservoir located past the southern end of Pukemapu Road. This is not seen as being a difficult project.

- Wastewater: A wastewater trunk main will need to be laid from Maleme Street to Pukemapu Road where a pump station will be required in the vicinity of the road bridge over Waimapu Stream. Various smaller trunk mains will then follow the gullies to service the adjacent housing developments. The sizing of these mains will be dependant upon decisions being made as to which of the potential upstream areas will be proceeded with i.e. Neewood Road, Ohauiti South and Kaitemako Road. No particular difficulties are expected with this work.

- Stormwater : Given the broken nature of this country, the location of stormwater ponds for treatment and detention of the increased runoff from development should not be difficult however there is likely to be more smaller ponds rather than a few large ponds.

- Roading: The major issue for the development of Pukemapu Road will be the affordability of the upgrade of the existing single lane bridge over the Waimapu Stream along with the upgrading of the road between Oropi Road and the bridge. The remainder of Pukemapu Road will also require upgrading to a residential standard as development occurs along the road. To enable this road and bridge to be constructed to the correct widths, a decision will need to be made at an early stage as to whether Pukemapu Road is to link through to Neewood Road. The development of Pukemapu Road and Neewood Road are quite straight forward in themselves but there is a section of approximately 0.9 Km. between them that will be more of a challenge to develop as a residential subdivision with earthwork cuts and fills in the order of 15m.

- Neewood Road.

- Water : Water supply will require an extension of the system serving Pukemapu Road up to a reservoir located in the vicinity of the Ohauiti Road/Rowe Road intersection. If the Pukemapu /Neewood Link was not constructed this water supply pipeline would need to be laid over some broken country but this is still relatively straight forward.

- Wastewater : Once the Wastewater trunk mains have been extended into the Neewood area from Pukemapu Road, the servicing of this area should be relatively easy. Two mains will be required, one following the Waimapu Stream and the other following the gully eastwards towards Ohauiti Road.

- Stormwater : As for Pukemapu Road, this land lends itself to the construction of more, smaller stormwater treatment ponds rather than a few larger ponds.

- Roading : Except for the 0.9 Km. link between Pukemapu Road and Neewood Road (discussed in C above), roading within the Neewood Road area is reasonably straight forward. The real issue is the affect that this development will have on Ohauiti Road and subsequently, the State Highway.

- Upper Ohauiti.

This land consists of several fingers of land left and right off Ohauiti Road. Since first considering this land for future development, one finger has been subdivided into “Rural Residential” blocks which will make future further development more difficult, and another finger has had a significant heritage site identified on it that will likely restrict residential development in that area. Closer to Neewood Road the land is already developed as a rural residential area with existing small orchard blocks. If the development of both Neewood Road and Upper Ohauiti were to create too much traffic for Ohauiti Road, this area could be forfeited in favour of Neewood Road.

- Water : Water supply would be from the Reservoir recently constructed on Ohauiti Road.

- Wastewater : Wastewater disposal would be by way of a connection to the Pukemapu/Neewood trunk main.

- Stormwater : As for Pukemapu Road, this land lends itself to the construction of more, smaller stormwater treatment ponds rather than a few larger ponds.

- Roading: All access would be by cul-de-sacs off Ohauiti Road.

- Kaitemako Road.

Little work has been done on the potential development of this land. On first inspection this land looked to have good development potential, however on more detailed investigation using a contour plan, much of this land is already at slopes of 1:4 and 1:5 and therefore too steep to develop further as residential lots. Of the 44 Ha. only 18 – 20 Ha. would be developable and this includes two knolls that look as though they have the potential to be archaeological sites. If this were the case, it would seriously limit any development on this site.

The land is isolated from other residential zones by 1.75 Km. of rural road and is 1Km south of Panorama Drive - a rural residential development. Some of the land between Panorama Drive and this site, on the eastern side of Kaitemako Road has potential for residential development but probably has nowhere to connect to a wastewater system. This particular site could be serviced for wastewater by a pump station and rising main connecting to the Pukemapu/Neewood system via the Upper Ohauiti reticulation. All of the land referred to in this area is multiply owned Maori land.

Given the small residential yield from the land and its isolation from other residential land, the site is probably better left in it’s current “Rural” state and removed from the SmartGrowth settlement pattern.

- Te Tumu.

Considerable work has been done towards the future Urban development of the Te Tumu land and this confirms that development of this land would be viable. The infrastructure issues are as follows:

- Water : Water supply is planned for Te Tumu and this will come from the future Waiari water treatment plant via a pipe laid along Bell Road

- Wastewater : There are a couple of options for the future provision of wastewater disposal facilities and these will be discussed further when considering the Generation 5 options, as they are interlinked.

- Stormwater : Stormwater is already covered with a Comprehensive Stormwater Consent which was negotiated as part of the Wairakei development.

- Roading : The principle vehicle access point will be via the planned interchange onto the Tauranga Eastern Motorway located in the South East corner of the Wairakei development. Other access will be via Te Okuroa Drive, The Boulevard and Papamoa Beach Road. A further bridge across the Kaituna River at the eastern end of the Te Tumu area, in the vicinity of Ford Road has been discussed but the need for this has not been confirmed at this stage.

# 6.2  Western Bay of Plenty

- Waihi Beach:

  - Wilson Park

  - Three Mile Creek

  - Athenree

- Water : The current water supply to Waihi Beach has the capacity to service all of the proposed future development.

- Wastewater : The current Discharge Consent for the Wastewater Treatment Plant expired in May 2012 and a new consent was applied for in November 2011. This new consent included an increase to the discharge quantity from 6500m³ per day to 8500m³ per day and is currently proceeding through the Environment Court. The proposed discharge quantity of 8500m³ will cater for a permanent population of 14,000 people and a summer peak of 21,900. The existing treatment plant has a capacity of 21,000 people and the current population is approximately 3,000 people.

- Stormwater : Stormwater treatment and detention can be achieved on this land.

- Roading : No significant roading issues have been identified in this area.

as a result of this proposed development

- Katikati

  - Busby Road.

  - Johnson Farm – Park Road

  - Tetley Road Industrial

    - Water : The water supply for Katikati comes from the Wharawhara valley and is capable of providing a supply for the proposed 2051 population of 9,210.

    - Wastewater : Wastewater disposal is an issue for Katikati. The current consent, which expires in 2016, allows for a discharge of 3000m³ per day and this is equates to a population of 4,000 people. Katikati’s population in the 2006 census was 3726. The wastewater is discharged to the ocean via a pipeline from the current treatment plant, across the harbour and Matakana Island to the ocean. This pipe is also running at its capacity at the moment and overflows in some storm events. Efforts are currently being made to flush this pipe and if this is successful it should result in a dramatic increase in the pipe’s capacity. Growth in Katikati will be governed by this pipe capacity. In the current situation, Katikati cannot grow any further however if the pipe flushing is successful then further development could be allowed but the amount would be dependant upon the results of the flushing. It is unlikely that a new discharge pipe to the ocean would be affordable to that portion of the community that would not be serviced by the existing pipe.

    - Given this problem it is unlikely that Katikati would be able to develop to the Smartgrowth population projection of 9,210 for 2051.

    - Stormwater : Stormwater treatment and detention ponds would be able to be constructed to service the proposed growth areas.

    - Roading : Once the extent of the allowable development is known from the wastewater work, negotiations with NZTA may be required to identify any works that may be required on the State Highway as a result of this growth with the associated cost sharing. Construction of the proposed Katikati Bypass will reduce some of the effect on the State Highway from local development.

- Omokoroa

  - Stage 2 (between the railway and SH No. 2)

Infrastructure planning for Omokoroa has largely been completed as part of the Stage 1 plan change.

- Water : The current water supply is from bores in the Whakamaramara/Youngson Road area and this, plus the possible need for a further bore in the area, is sufficient to meet the requirements of the fully developed area.

- Wastewater : A wastewater pipe has been laid from Omokoroa to Tauranga to cater for the planned capacity of Stages 1 & 2 of the Omokoroa development which are predicted to have a population equivalent of 12,000 people.

- Stormwater : The topography of Omokoroa will allow various methods of stormwater treatment and detention to be employed including low impact design.

- Roading : A major issue to be addressed in the future is the need to upgrade the existing access onto SH No. 2, as well as the possibility of upgrading various parts of the State Highway itself. Some discussions have been held with NZTA at the time of rezoning Omokoroa Stage 1 and WBOPDC were to investigate the possibility of a Park and Ride station in the vicinity of the intersection as part of their mitigation of the traffic effects. No decision has yet been reached on this option.

- Te Puke

  - Cannel Farm (South)

  - No. 3 Road Industrial Land.

The 2006 Census population for Te Puke was 7080 people and the SmartGrowth projected population out to 2051 is a further 5920 giving a total population of 13,000 at 2051.

- Water : The water supply for Te Puke comes from bores in No. 1 Road and this is sufficient service the town in the foreseeable future. Any augmentation that may be required in the future will come from the planned Waiari supply.

- Wastewater : The current Wastewater discharge consent allows a discharge to the Waiari Stream of up to 9,000m³ per day (12,000 people) and expires in 2016. However the current treatment plant is designed for a population of 9,000 people (2006 Census was 7080 for Te Puke) and an upgrade to the plant is planned that will increase its discharge up to the maximum consented rate. There is a small discrepancy between the maximum consented discharge catering for a population of 12,000 and the SmartGrowth projected population of 13,000 and this would need to be addressed when applying for a new consent in 2016, or designing the proposed treatment plant expansion. Note that these comments are on the basis that the existing discharge consent will be renewed and that wastewater from Rangiuru is not included in the new consent application.

- Stormwater : The topography of this land will encourage the use of low impact design stormwater treatment which would minimise the size of any ponds that may be required.

- Roading : The completion of the TEL motorway will change the function of SH2 through Te Puke, NZTA is conducting a post-TEL form and function review of this corridor as part of the development of 30 year strategy for it. The further growth planned for Generation 4 development is likely to result in the need to upgrade the Te Puke Quarry Road, No. 3 Road and No. 1 Road intersections onto SH. No.2.

- CONCLUSIONS.

Conclusions to be drawn from this report are:

- Development of the Tauriko Stage 2 area will be restricted until 2016 when the Southern Pipeline and the second connection to the water supply are completed.

- Development of Keenan Road is dependant upon cost effective wastewater and roading solutions being achieved.

- A decision needs to be made as to whether Pukemapu and Neewood roads need to be linked.

- Pukemapu Road development will depend upon the viability of upgrading the Waimapu Bridge and connecting road from the bridge to Oropi Road and whatever upgrading may be required to the Oropi Road/SH No.29 intersection. This would be assisted if the link between Pukemapu Road and Neewood Road were made.

- Neewood Road could be viable but would be assisted by the development of Pukemapu Road first, thus facilitating more cost effective wastewater services.

- Upper Ohauiti does not have the same yield potential as Neewood Road and this area combined with Neewood Road could create traffic issues on Ohauiti Road and consequently the State Highway. If these problems prove significant, this area could be forfeited in favour of Neewood Road.

- Kaitemako Road land is steeper than first envisaged and the potential yield is low. It is quite isolated from other residential development and with the low yield is likely to be expensive to develop. This land is best left as “Rural” land and should be removed from the SmartGrowth settlement Pattern.

- Work done on the Te Tumu area shows that it is viable to develop.

- Waihi Beach. No impediments are envisaged for the development of this land.

- Katikati. The major impediment to the further development of Katikati is the capacity of the wastewater ocean outfall pipe. Given the current situation, the existing population exceeds the capacity of this pipe during storm events but work is being done to flush the pipe which has the potential to vastly improve its capacity.

- Water and wastewater services are available for the development of Omokoroa Stage 2 but the roading connection to State Highway No. 2 is still to be resolved.

- An upgrade of the Te Puke wastewater treatment plant is planned to increase its capacity to the maximum consented discharge for the equivalent of 12,000 people. SmartGrowth’s current projected population for the town is 13,000 people. NZTA is currently conducting a post TEL form and function review for the roading corridor through Te Puke.